| Nr | Date | Ref. Change Request | Description | Implementation |
| --- | --- | --- | --- | --- |
| 1 | 21/10/02 | PI-OPS-015 | Set new limits for HV power supply temperatures: L5379 and L5380 | IODB updated to set the new limits TM Paramter; OOL Cond.; Hard Low; Soft Low; Soft High; Hard High. L5379; X8508; -35 Deg C; -30 Deg C; 75 Deg C; 80 Deg C. L5380; X8508; -35 Deg C; -30 Deg C; 75 Deg C; 80 Deg C. |
| 2 | 22/10/02 | PI-OPS-020 | Enable automatic dump of the HSL buffer when OEM 154 is generated. | Send TC L0034 L0160 = ENABLED [1] To enable automatic dump of the HSL buffer when OEM 154 (HSL loss of synchronization) is generated. One APID 1542 is generated for each OEM 154. Procedure FCP_JEM2_0015 modified to uplink TC upon activation of the IASW to have this feature always enabled. FCP_JEM2_0015, Issue 1.3, step 7. |
| 3 | 27/10/02 | PI-OPS-033 | Set thresholds for Radiation Monitoring Count Rates (RMCR) | Execute FCP_JEM2_0140 with TC L0039 L0167: RMC#1 ENTER = 2 L0168: RMC#1 EXIT = 1 L0169: RMC#2 ENTER = 60 L0170: RMC#2 EXIT = 50 L0171: RMC#3 ENTER = 2 L0172: RMC#3 EXIT = 1 |
| 4 | 29/10/02 | PI-OPS-060 | Reduction of nominal HV setting for JEM-X2 from 89 Dec to 88 Dec. JEM-X2 reaches the nominal gas-gain at lower voltages. | Set new data-base default values for TC parameters: TC LU0003A L0001 = 85 [Dec Raw] And TC LU0003B L0001 = 88 [Dec Raw] Update of procedures LEHVON01, LEHVAJ01 and FCP_JEM2_1010 with the new command parameter values |
| 5 | 30/10/02 | PI-OPS-061 | Change thresholds for Radiation Monitoring Count Rates (RMCR) | Execute FCP_JEM2_0140 with TC L0039 L0167: RMC#1 ENTER = 0 L0168: RMC#1 EXIT = 1 L0169: RMC#2 ENTER = 60 L0170: RMC#2 EXIT = 50 L0171: RMC#3 ENTER = 0 L0172: RMC#3 EXIT = 1 Update of FCP_JEM2_0015 to set the new RMCR thresholds after activation of the IASW. FCP_JEM2_0015, Issue 1.3, step 6.7 and 6.8. |
| 6 | 30/10/02 | PI-OPS-061 | Set new values for the Radiation Monitoring Delays: HL = 40 sec LL = 1600 sec | Execute FCP_JEM2_0142 with TC L0054 L0175 = 40 sec L0176 = 1600 sec Update of FCP_JEM2_0015 to include step for setting the RMCR delays. FCP_JEM2_0015, issue 1.3, steps 6.11 and 6.12. Update in FCP_JEM2_0017, issue 1.3, step 4.1. |
| 7 | 09/11/02 | PI-OPS-116 | Change the S/W HV switch OFF limit to 20000 triggers per 8 seconds. | Execute FCP_JEM2_0050 with TC L0017 L0017 = A08A Hex L0179 = 20000 dec |
| 8 | 12/11/02 | PI-OPS-136 | Reduction of nominal HV setting for JEM-X2 from 88 Dec to 80 Dec. | Set new data-base default values for TC parameters: TC LU0003A L0001 = 77 [Dec Raw] And TC LU0003B L0001 = 80 [Dec Raw] Update of procedures LEHVON01, LEHVAJ01 and FCP_JEM2_1010 with the new command parameter values |
| 9 | 25/11/02 | PI-OPS-165 | Set the maximum acceptable anode signal to 800 dec | Execute FCP_JEM1_0070 with: TC L0014 L0013 = 80 sec L0014 = 800 sec Update FCP_JEM2_0025, issue 1.3, steps 3.7 and 3.9. |
| 10 | 27/11/02 | PI-OPS-166 | Uplink of new Energy Linearisation Table | Update of the data-base with the new Energy Liniearisation Table and uplink of the table: 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32 33 34 35 36 37 38 39 40 41 42 43 44 45 46 47 48 49 50 51 52 53 54 55 56 57 58 59 60 61 62 63 64 65 66 67 68 69 70 71 72 73 74 75 76 77 78 79 80 81 82 83 84 85 86 87 88 89 90 91 92 93 94 95 96 97 98 99 100 101 102 103 104 105 106 107 108 109 110 111 112 113 114 115 116 117 118 119 120 121 122 123 124 125 126 127 128 130 132 134 136 138 140 142 144 146 148 150 152 154 156 158 160 162 164 166 168 170 172 174 176 178 180 182 184 186 188 190 192 196 200 204 208 212 216 220 224 228 232 236 240 244 248 252 256 260 264 268 272 276 280 284 288 292 296 300 304 308 312 316 320 328 336 344 352 360 368 376 384 392 400 408 416 424 432 440 448 456 464 472 480 488 496 504 512 520 528 536 544 552 560 568 576 592 608 624 640 656 672 688 704 720 736 752 768 784 800 816 832 848 864 880 896 912 928 944 960 976 992 1008 1024 1040 1056 4095 Add step to FCP_JEM2_0020 to request uplink of the latest Energy Linearisation Table after activation of the DFEE for the first time after DPE activation. FCP_JE21_0020, Issue1.3, step 6. |
| 11 | 25/11/02 | PI-OPS-167 | Update of JEM-X2 DPE S/W (version 0.0.2) | OBSMS tool/procedure was used to loaded JEM-X2 DPE image: # IIMG_P_JM2_0129_00006_F_X_000_000_2.INT The patch should prevent data losses, which occur because of HSL loss of synchronization. OEM154 will still be generated. The image also contains the correction of the IASW interpretation of the pointing ID => FCP_JEM2_0011. Step 11 added to FCP_JEM2_0010 to load the latest DPE version using the OBSMS after DPE activation. |
| 12 | 25/11/02 | PI-OPS-168 | Update of JEM-X2 DFEE S/W (version 4.2.0) | JEM-X procedure FCP_JEM2_0025 was used to patch the changes to the DFEE S/W corresponding to the DFEE image: # IIMG_P_J2D_1665_00255_F_V_004_002_0.IMG The database default values of the relevant TC parameters of procedure FCP_JEM2_0025 were changed to the ones provided by the JEM-X PI. After the DB update/propagation, procedure FCP_JEM2_0025 was executed to load the new DFEE S/W version 4.2. The file below contains the DFEE SW change from version 4.1 to 4.2. The image also contains the change implemented by CR PI-OPS-116: “Change the S/W HV switch OFF limit to 20000 triggers per 8 seconds.” Procedures FCP_JEM2_0020 and FCP_JEM2_0025 updated to indicate the new DFEE SW version 4.2. |
| 13 | 28/11/02 | PI-OPS-191 | Change the nominal Low Level Discriminator to 2 | The operational nominal value of the Low Level Discriminator changed from 10 to 2. Procedures FCP_JEM2_1010, LEACAL01 and LEHVON01 updated to implement the change. |
| 14 | 05/12/02 | N/A | Change of PST windows for JEM-X | The fixed part of the PST is changed to allocated only 1 PST window (previous value was 3) to JEM-X2. The freed PST windows are available in the variable part of the PST for ISOC to assign to different instruments as required. Reference: INT-MOC-SYS-TN-1016-TOS-OF |
| 15 | 15/01/03 |  | Change of the OOL checking condition for HV Power supply temperatures. | The OOL condition for parameters L5379 and L5380 is changed to prevent OOL alarm when the DFEE is OFF: 1- Remove the out-of-limit check for the condition when the DFEE is OFF (L5022=0) 2- Assign the DFEE ON condition (L5022 >0) as the validity condition for these parameters. |
| 17 | 03/02/03 | PI-OPS-214 | Change the nominal Low Level Discriminator to 5 | The operational nominal value of the Low Level Discriminator is changed from 2 to 5. Procedures FCP_JEM2_1010, LEACAL01 and LEHVON01 updated to implement the change. |
| 18 | 06/02/03 | PI-OPS-213 | Update of JEM-X2 DFEE S/W (4.3.0) | Three new JEM-X2 DFEE patch TCs are defined in the DB to patch the SW upgrade from version 4.2.0 to 4.3.0, corresponding to DFEE image file: IIMG_P_J2D_1665_00255_F_V_004_003_0.IMG The file below contains the new TCs for DFEE SW upgrade from version 4.2.0 to 4.3.0: The patch modifies the event handling routine in order to reduce the dead time of the instrument. This is done through quick rejection of high amplitude signals during read-in of data by comparing the energy signal with a command able limit that is stored in the SW integer parameter #81. The patch also changes the electronic calibration levels. Procedure FCP_JEM2_0025, and its TC sequence updated (issue 1.4) to include the three newly defined JEM-X2 DFEE patch TCs. |
| 19 | 06/02/03 | PI-OPS-213 | Uplink of new value for SW integer parameter #81 | Procedure FCP-JEM2_0070 executed to set the new value of SW integer parameter #81 to 1214 decimal. FCP_JEM2_0025 updated (issue 1.4) with the new value of parameter #81. |
| 20 | 25/02/03 | PI-OPS-231 | Update of JEM-X2 DFEE S/W (4.4.0) | JEM-X procedure FCP_JEM2_0025 was used to patch the changes to the DFEE S/W corresponding to the DFEE image: # IIMG_P_J2D_1665_00255_F_V_004_004_0.IMG The database default values of the relevant TC parameters of procedure FCP_JEM2_0025 were changed to the ones provided by the JEM-X PI. The file below contains the DFEE SW change from version 4.3 to 4.4: The S/W change should improve the instrument sensitivity at energies below 8 keV by updating the onboard event selection criteria (tables). |
| 21 | 16/04/03 | PI-OPS-242 | Update of S/W integer parameters #80 and #81 | Procedure FCP-JEM2_0070 was executed to set the value of SW integer parameter #80 to 1000 decimal and the value of SW integer parameter #81 to 1414 decimal. The change will increase the upper limits for the raw anode signals to be transmitted from JEM-X2. FCP_JEM2_0025 updated (issue 1.5) with the new value of parameters #80 and #81. |
| 22 | 16/04/03 | PI-OPS-243 | Reduction of nominal HV setting for JEM-X2 from 80 Dec to 79 Dec. | Set new data-base default values for TC parameters: TC LU0003A L0001 = 76 [Dec Raw] And TC LU0003B L0001 = 79 [Dec Raw] Update of procedures LEHVON01, LEHVAJ01 and FCP_JEM2_1010 with the new command parameter values |
| 23 | 01/05/03 | PI-OPS-243 | Reduction of nominal HV setting for JEM-X2 from 79 Dec to 78 Dec. | Set new data-base default values for TC parameters: TC LU0003A L0001 = 75 [Dec Raw] And TC LU0003B L0001 = 78 [Dec Raw] Update of procedures LEHVON01, LEHVAJ01 and FCP_JEM2_1010 with the new command parameter values |
| 24 | 04/05/03 | INT-OR-0066 Based on Email from R.Much | Change thresholds for Radiation Monitoring Count Rates (RMCR) | Execute FCP_JEM2_0140 with TC L0039 L0167: RMC#1 ENTER = 20 L0168: RMC#1 EXIT = 10 L0169: RMC#2 ENTER = 60 L0170: RMC#2 EXIT = 50 L0171: RMC#3 ENTER = 64 L0172: RMC#3 EXIT = 25 Update of FCP_JEM2_0015 to set the new RMCR thresholds after activation of the IASW. FCP_JEM2_0015, Issue 1.4, step 6.7 and 6.8. |
| 25 | 04/05/03 | INT-OR-0066 Based on Email from R.Much | Set new values for the Radiation Monitoring Delays: HL = 16 sec LL = 1800 sec | Execute FCP_JEM2_0142 with TC L0054 L0175 = 16 sec (2 decimal raw) L0176 = 1800 sec (225 decimal raw) Update of FCP_JEM2_0015, issue 1.4, steps 6.11 and 6.12. Update in FCP_JEM2_0017, issue 1.4, step 4.1. |
| 26 | 13/05/03 | INT-OR-0069 Based on PI-OPS-246 | Change thresholds for Radiation Monitoring Count Rates (RMCR) | Execute FCP_JEM2_0140 with TC L0039 L0167: RMC#1 ENTER = 20 L0168: RMC#1 EXIT = 10 L0169: RMC#2 ENTER = 60 L0170: RMC#2 EXIT = 50 L0171: RMC#3 ENTER = 64 L0172: RMC#3 EXIT = 50 Update of FCP_JEM2_0015 to set the new RMCR thresholds after activation of the IASW. FCP_JEM2_0015, Issue 1.4, step 6.7 and 6.8. |
| 27 | 13/05/03 | INT-OR-0069 Based on PI-OPS-246 | Set new values for the Radiation Monitoring Delays: HL = 16 sec LL = 600 sec | Execute FCP_JEM2_0142 with TC L0054 L0175 = 16 sec (2 decimal raw) L0176 = 600 sec (75 decimal raw) Update of FCP_JEM2_0015, issue 1.4, steps 6.11 and 6.12. Update in FCP_JEM2_0017, issue 1.4, step 4.1. |
| 28 | 14/05/03 |  | ODB Update: Change of Hard High OOL on RAD Mon parameters | 1- Change the High Hard Monitoring check (High Hard OOL) of parameter L5315 to 19 2- Change the High Hard Monitoring check (High Hard OOL) of parameter L5316 to 59 3- Change the High Hard Monitoring check (High Hard OOL) of parameter L5317 to 63 Changes will be in effect with the next ODB propagation (as of 15/05/03) |
| 29 | 14/05/03 |  | ODB Update: New monitoring checks (OOL) for the S/W Trigger Rate | Introduce the following monitoring check (OOL) for parameter L5119: High Soft Limit = 15000 High Hard Limit = 19000 Number of Checks (OCF_NBCHCK) = 1 Changes will be in effect with the next ODB propagation (as of 15/05/03) |
| 30 | 15/07/03 | OCR - 148 | Reduction of nominal HV setting for JEM-X2 from 78 Dec to 77 Dec. | Set new data-base default values for TC parameters: TC LU0003A L0001 = 74 [Dec Raw] And TC LU0003B L0001 = 77 [Dec Raw] Update of procedures LEHVON01, LEHVAJ01 and FCP_JEM2_1010 with the new command parameter values |
| 31 | 15/07/03 | OCR – 149 | Update of S/W integer parameters #80 and #81 | Procedure FCP-JEM2_0070 was executed to set the value of SW integer parameter #80 to 1200 decimal and the value of SW integer parameter #81 to 1614 decimal. The change will increase the upper limits for the raw anode signals to be transmitted from JEM-X2. FCP_JEM2_0025 updated (issue 1.5) with the new value of parameters #80 and #81. |
| 32 | 01/08/03 | N/A (suggested in real time by PI) | Temporary increase of the S/W HV switch OFF limit to 30000 triggers per 8 seconds. This was to avoid that JEM-X2 goes to Safe mode during Sco-X1 Diagnostic observation | At 2003-08-01T21:35Z: Executed FCP_JEM2_0050 with TC L0017 L0017 = A08A Hex L0179 = 30000 dec At 2003-08-02T00:53Z: Executed FCP_JEM2_0050 with TC L0017 L0017 = A08A Hex L0179 = 20000 dec |
| 33 | 11/08/03 | OCR - 152 | Change the S/W HV switch OFF limit to 25000 triggers per 8 seconds. | Execute FCP_JEM2_0050 with TC L0017 L0017 = A08A Hex L0179 = 25000 dec |
| 34 | 12/10/03 (IODB update 28/10/03) | OCR – 155 | Reduction of nominal HV setting for JEM-X2 from 77 Dec to 76 Dec. | Set new data-base default values for TC parameters: TC LU0003A L0001 = 73 [Dec Raw] And TC LU0003B L0001 = 76 [Dec Raw] Update of procedures LEHVON01, LEHVAJ01 and FCP_JEM2_1010 and LEACT_02 with the new command parameter values and CEV/PTV conditions. |
| 35 | 24/10/03 (IODB update 28/10/03) |  | IODB Update: Change of condition of OOL on RAD Mon parameters | Change monitoring check conditions for radiation monitor parameters as follows: L5315 (RAD MONITOR 1) monitoring check condition: LD5322D.raw = 1 [TRUE] L5316 (RAD MONITOR 2) monitoring check condition: LD5322D.raw = 1 [TRUE] L5317 (RAD MONITOR 3) monitoring check condition: LD5322D.raw = 1 [TRUE] where LD5322D = (L5322.raw = 0 [regard RMF] AND GD0028D.raw = 1 [OUT RAD BELT]) |
| 36 | 11/11/03 (IODB update 14/11/03) |  | IODB Update: Change of condition of OOL on RAD Mon parameters | Change definition of derived parameter used in monitoring check condition for radiation monitor parameters: LD5322D = (L5322.raw = 0) [regard RMF] AND (L5022.raw > 1) [i.e. DFEE not OFF or in SAFE State] Also, set Number of Checks (OCF_NBCHCK) =1 for monitoring checks of radiation monitor parameters: L5315 (RAD MONITOR 1) L5316 (RAD MONITOR 2) L5317 (RAD MONITOR 3) |
| 37 | 20/11/03 | OCR – 159 | JEM-X2 Calibration Source Engineering Session | As described in INT_OR_0114, JEM-X2 was operated in a special configuration between 2003-11-20T13:04:00Z and 2003-11-20T17:41:45Z in accordance with OCR-159 (i.e. with DFEE S/W Integer Parameter 64 set to 1023dec). |
| 38 | 16/03/04 | INTGSCCR-45 and INTGSCCR-46 | Update of JEM-X2 DFEE S/W (5.3.0) | JEM-X procedure FCP_JEM2_0025 was used to patch the changes to the DFEE S/W corresponding to the DFEE image: # IIMG_P_J2D_1665_00255_F_V_005_001_0.INT The database default values of the relevant TC parameters of procedure FCP_JEM2_0025 were changed to the ones provided by the JEM-X PI. The file below contains the DFEE SW change from version 4.4 to 5.3: The S/W change should improve the signal to noise ratio, particularly at energies above 15keV by updating the on-board background rejection criteria. In addition the on-board safety limit for HV commanding was updated. |
| 39 | IODB update 16/03/04 | OCR-168 | Reduction of nominal HV setting for JEM-X2 from 76 Dec to 74 Dec. | Set new data-base default values for TC parameters: TC LU0003A L0001 = 71 [Dec Raw] And TC LU0003B L0001 = 74 [Dec Raw] Update of procedures LEHVON01, LEHVAJ01 and FCP_JEM2_1010 and LEACT_02 with the new command parameter values and CEV/PTV conditions. |